SECTION 02100

SITE PREPARATION

PART 1	GENERAL

1.01		DESCRIPTION

A.	Perform site preparation work as shown and specified.  The work includes:

- Protecting existing trees, plants, and improvements to remain.

- Removing vegetation, debris, and waste materials.

1.02		QUALITY ASSURANCE

A.	Comply with Section 02000 requirements.

1.03		PROJECT CONDITIONS

A.	Perform site preparation work before commencing site construction.

B.	Locate, protect and maintain active utilities and site improvements to remain.

C.	Restore to original grades and conditions areas adjacent to site disturbed or damaged as a result of site preparation work.

PART 2	PRODUCTS

2.01		MATERIALS

A.	Materials and Equipment: As selected by the Contractor, except as indicated.

B.	Tree Protection:

1.	Temporary Fencing

- Stockpiling of any materials under dripline of a tree is not permitted.

PART 3	EXECUTION

3.01		CLEARING

A.	Protect existing trees scheduled to remain against injury or damage, including cutting, breaking or skinning of roots, trunks or branches, smothering by stockpiled construction materials, excavated materials, equipment or vehicular traffic within the branch spread.  Fires are not permitted.

1.	Protect designated trees with temporary fence enclosure.  Provide a minimum 8’-0” radius from the center of the tree trunk.  Increase enclosure size as directed for large trees.

2.	Erect fencing before commencing site preparation work.  Maintain fencing during full construction period.  Remove when no longer needed or when acceptable to the City Representative.

3.	Interfering branches of trees to remain may be removed when acceptable to the City Representative. All pruning performed shall preserve the branch collar to facilitate wound healing (see Section 02490: Trees). The Contractor shall use no tree paint unless directed to do so by the City Representative.

4.	Repair trees damaged by construction operations in a manner acceptable to the City Representative.  Repair damaged trees promptly to prevent progressive deterioration caused by damage.

5.	Replace trees damaged beyond repair by construction operations, as determined

by the City Representative, with trees of similar size and species.  Cost for tree replacement shall be determined in accordance with the Tree Evaluation Formula as described in:  A Guide to the Professional Evaluation of Landscape Trees, Specimen Shrubs and Evergreens, current edition,  published by the International Society of Arboriculture.  Repair and replacement of trees scheduled to remain and damaged by construction operations or lack of adequate protection during construction operations shall be at the Contractor's expense.

3.02	STRIPPING TOPSOIL

A.	Strip topsoil to its full depth at all areas to be regarded, resurfaced, or paved within contract limit work area.

B.	Stop topsoil stripping at trees designated to remain sufficient distance to prevent damage to the root system.

C.	Remove topsoil from indicated areas and replace, fine grade, and seed in park as shown and to the satisfaction of the City Representative.

D.	Protect all areas which are not to be resurfaced or regarded, and adjacent areas outside of the contract limits from damage due to site preparation work.

3.03	DISPOSAL OF WASTE MATERIALS

A.	Stockpile, haul from site and legally dispose of waste materials and debris.  Accumulation is not permitted.

B. 	Maintain disposal routes clear, clean and free of debris.

C.	Onsite burning of materials is not permitted.

D.	Existing Utilities:

1.	Information on the drawings relating to existing utility lines and services is from the best sources presently available.  All such information is furnished only for information and is not guaranteed.  Excavate test pits as required to determine exact locations of existing utilities.

2.	Contact Ohio Utilities Protection Service at 1-800-362-2764 twenty-four hours prior to digging.

3.	Perform work and provide necessary materials to disconnect or relocate existing utilities as necessary.  Record utility termination points before disconnecting.

3.04	CLEANING

A.	Upon completion of site preparation work, clean areas within contract limits, remove tools and equipment.  Provide site clear, clean and free of materials and debris suitable for site work operations.

END OF SECTION